Configurations OSPF Phase 6 








FW-NY#show running-config

Building configuration...

Current configuration : 1310 bytes

!

version 15.4

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname FW-NY

!

!

!

!

!

!

!

!

ip cef

no ipv6 cef

!

!

!

!

!

!

!

!

!

!

no ip domain-lookup

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface GigabitEthernet0/0/0

no ip address

duplex auto

speed auto

shutdown

!

interface GigabitEthernet0/0/1

no ip address

duplex auto

speed auto

shutdown

!

interface GigabitEthernet0/0/2

no ip address

duplex auto

speed auto

shutdown

!

interface Serial0/1/0

ip address 172.16.0.6 255.255.255.252

!

interface Serial0/1/1

ip address 172.16.0.10 255.255.255.252

!

interface Serial0/2/0

description *** WAN to FW-BRU ***

ip address 172.16.0.21 255.255.255.252

clock rate 2000000

!

interface Serial0/2/1

no ip address

clock rate 2000000

shutdown

!

interface Vlan1

no ip address

shutdown

!

router ospf 10

router-id 1.1.1.1

log-adjacency-changes

passive-interface default

no passive-interface Serial0/1/0

no passive-interface Serial0/1/1

no passive-interface Serial0/2/0

network 172.16.0.20 0.0.0.3 area 0

network 172.16.0.4 0.0.0.3 area 0

network 172.16.0.8 0.0.0.3 area 0

!

ip classless

ip route 0.0.0.0 0.0.0.0 172.16.0.22

!

ip flow-export version 9

!

!

!

!

!

!

!

line con 0

!

line aux 0

!

line vty 0 4

login

!

!

!

end

FW-NY#

FW-BRU#show running-config

Building configuration...

Current configuration : 1173 bytes

!

version 15.4

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname FW-BRU

!

!

!

!

!

!

!

!

ip cef

no ipv6 cef

!

!

!

!

!

!

!

!

!

!

no ip domain-lookup

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface GigabitEthernet0/0/0

ip address 172.16.0.14 255.255.255.252

duplex full

speed 100

!

interface GigabitEthernet0/0/1

ip address 172.16.0.18 255.255.255.252

duplex full

speed 100

!

interface GigabitEthernet0/0/2

no ip address

duplex auto

speed auto

shutdown

!

interface Serial0/1/0

description *** WAN to FW-NY ***

ip address 172.16.0.22 255.255.255.252

!

interface Serial0/1/1

no ip address

clock rate 2000000

shutdown

!

interface Vlan1

no ip address

shutdown

!

router ospf 10

router-id 2.2.2.2

log-adjacency-changes

passive-interface default

no passive-interface GigabitEthernet0/0/0

no passive-interface GigabitEthernet0/0/1

no passive-interface Serial0/1/0

network 172.16.0.20 0.0.0.3 area 0

network 172.16.0.12 0.0.0.3 area 0

network 172.16.0.16 0.0.0.3 area 0

!

ip classless

!

ip flow-export version 9

!

!

!

!

!

!

!

line con 0

!

line aux 0

!

line vty 0 4

login

!

!

!

end

FW-BRU#